Technical data
Mercedes-Maybach S 680 Saloon

As of: 08 April 2026

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | V8 |
| Displacement | cc | 3,982 |
| Hub x bore | mm x mm | 92.00 x 83.00 |
| Rated output1 | kW [hp] at rpm | 450 [612]/5,500-6,100 |
| Rated torque2 | Nm at rpm | 850/2,500-4,500 |
| Compression ratio |  | 10.50 : 1 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric)3 | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 390 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Ratios (1/2/3/4/5/6/7/8/9/R1) |  | 5,35/3,24/2,25/1,64/1,21/1,00/0,87/0,72/0,60/R1 4,80 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,484/1,921/1,510 |
| Wheelbase | mm | 3,396 |
| Turning circle | m | 12.40 |
| Track width, front/rear | mm | 1,655/1,683 |
| Luggage compartment capacity, VDA4 | l | 480 |
| Kerb weight/payload/permissible gross mass5 | kg | 2,430 |
| Tank capacity/reserve | l | 76/8 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.0 |
| Top speed | km/h | 250 |
| Energy consumption (combined)6 | l/100 km | 11.7 - 11.0 |
| CO₂ emissions (combined)6 | g/km | 267 - 252 |
| CO₂ class6 |  | G |

1 The specified rated power corresponds to the continuously available engine output defined in accordance with DIN 70020, ISO 14396 and ECE Regulation No. 24 under standardised test conditions. The power output is reproducible under standardised temperature, load and ambient conditions and represents the continuously available power output of the combustion engine.

2 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

3 The specified maximum power describes the peak power of the electric drive that is briefly available under the test conditions of ECE Regulation No. 85. This peak power is only available for limited periods and may vary depending on temperature, charge, conditioning and environmental conditions. The actual available power may differ from the specified maximum value depending on operating conditions.

4 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

5 Kerb weight, ready-to-drive in accordance with EU.

6 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.