Technical Data
Mercedes-Benz S 350 d 4MATIC Saloon, long version

As of: 18 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,989 |
| Rated output1 | kW [hp] at rpm | 230 [313]/3600 - 4800 |
| Rated torque1 | Nm at rpm | 650/1200 - 3200 |
| Fuel mix preparation |  | Injection pump/injection system model: 2-plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 320 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8 J x 18 ET 29.0 / 8 J x 18 ET 29.0 |
| Tyres, front/rear |  | 235/55 R18 / 235/55 R18 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,921/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.90 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 510 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,250 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |

| Driving performance, consumption and range |  |  |
| --- | --- | --- |
| Acceleration 0 - 100 km/h | s | 5.4 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 6.9 - 6.2 |
| CO₂ emissions (combined)4 | g/km | 182 - 163 |
| CO₂ class4 |  | G - F |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 350 d 4MATIC Saloon

As of: 18 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,989 |
| Rated output1 | kW [hp] at rpm | 230 [313]/3600 - 4800 |
| Rated torque1 | Nm at rpm | 650/1200 - 3200 |
| Fuel mix preparation |  | Injection pump/injection system model: 2-plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 320 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8 J x 18 ET 29.0 / 8 J x 18 ET 29.0 |
| Tyres, front/rear |  | 235/55 R18 / 235/55 R18 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.60 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 510 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,230 |
| Tank capacity/reserve | l | 76/8 |

| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| --- | --- | --- |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 5.4 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 6.8 - 6.2 |
| CO₂ emissions (combined)4 | g/km | 179 - 162 |
| CO₂ class4 |  | G - F |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 450 4MATIC Saloon, long version

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 280 [381]/5.000 - 6.100 |
| Rated torque1 | Nm at rpm | 560/2000 - 4500 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 320 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8 J x 18 ET 29.0 / 8 J x 18 ET 29.0 |
| Tyres, front/rear |  | 235/55 R18 / 235/55 R18 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,954/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.90 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 530 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,185 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.9 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 9.0 - 8.2 |
| CO₂ emissions (combined)4 | g/km | 206 - 186 |
| CO₂ class4 |  | G |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 450 4MATIC Saloon

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 280 [381]/5.000 - 6.100 |
| Rated torque1 | Nm at rpm | 560/2000 - 4500 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 320 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8 J x 18 ET 29.0 / 8 J x 18 ET 29.0 |
| Tyres, front/rear |  | 235/55 R18 / 235/55 R18 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.60 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 530 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,165 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.9 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 8.9 - 8.1 |
| CO₂ emissions (combined)4 | g/km | 203 - 185 |
| CO₂ class4 |  | G |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 450 d 4MATIC Saloon, long version

As of: 18 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,989 |
| Rated output1 | kW [hp] at rpm | 270 [367]/4000 - 4000 |
| Rated torque1 | Nm at rpm | 750/1350 - 2800 |
| Fuel mix preparation |  | Injection pump/injection system model: 2-plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 320 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,921/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.90 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 510 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,255 |
| Tank capacity/reserve | l | 76/8 |

| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| --- | --- | --- |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 5.1 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 6.9 - 6.3 |
| CO₂ emissions (combined)4 | g/km | 182 - 165 |
| CO₂ class4 |  | G - F |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 450 d 4MATIC Saloon

As of: 18 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,989 |
| Rated output1 | kW [hp] at rpm | 270 [367]/4,000 |
| Rated torque1 | Nm at rpm | 750/1,350 – 2,800 |
| Fuel mix preparation |  | Injection pump/injection system model: 2-plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 320 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.60 |
| Track width, front/rear | mm | 1,660/1,683 |
| Luggage compartment capacity, VDA2 | l | 510 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,235 |
| Tank capacity/reserve | l | 76/8 |

| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| --- | --- | --- |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 5.1 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 6.9 - 6.3 |
| CO₂ emissions (combined)4 | g/km | 179 - 164 |
| CO₂ class4 |  | G - F |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 450 e Saloon with EQ Hybrid technology, long version

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x2 (RWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 240 [326]/4,250 – 6,100 |
| Rated torque1 | Nm at rpm | 540/2,200 – 4,250 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 120 [163] |
| Rated torque (peak) (electric) | Nm | 480 |
| Rated output system output | kW [hp] | 320 [435] |
| System torque | Nm | 680 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,921/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.70 |
| Track width, front/rear | mm | 1,655/1,651 |
| Luggage compartment capacity, VDA2 | l | 345 |

| Kerb weight/payload/permissible gross mass3 | kg | 2,465 |
| --- | --- | --- |
| Tank capacity/reserve | l | 67/8 |
| HV battery - usable energy content | kWh | 21.96 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 5.7 |
| Top speed | km/h | 250 |
| Energy consumption (weighted, combined)4 | kWh/100km plus l/100 km | 18.0-16.6 3.1 - 2.2 |
| Fuel consumption with discharged battery (combined)4 | l/100 km | 8.7 - 7.5 |
| CO₂ emissions (weighted, combined)4 | g/km | 71 - 51 |
| CO₂ class4 |  | B / G – F |
| Electric range (EAER)4 | km | 95 – 117 |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 Figures are preliminary and were determined internally in accordance with the “WLTP test procedure” certification method. At present, there has been neither an EC type approval nor a certificate of conformity with official values. There may be discrepancies between the figures provided and the official values. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 450 e Saloon with EQ Hybrid technology

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x2 (RWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 240 [326]/4,250 – 6,100 |
| Rated torque1 | Nm at rpm | 540/2,200 – 4,250 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 120 [163] |
| Rated torque (peak) (electric) | Nm | 480 |
| Rated output system output | kW [hp] | 320 [435] |
| System torque | Nm | 680 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.40 |
| Track width, front/rear | mm | 1,655/1,651 |
| Luggage compartment capacity, VDA2 | l | 345 |

| Kerb weight/payload/permissible gross mass3 | kg | 2,445 |
| --- | --- | --- |
| Tank capacity/reserve | l | 67/8 |
| HV battery - usable energy content | kWh | 21.96 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 5.7 |
| Top speed | km/h | 250 |
| Energy consumption (weighted, combined)4 | kWh/100km plus l/100 km | 17.8-16.5 2.9 – 2.1 |
| Fuel consumption with discharged battery (combined)4 | l/100 km | 8.6 - 7.4 |
| CO₂ emissions (weighted, combined)4 | g/km | 69 - 50 |
| CO₂ class4 |  | B / G - F |
| Electric range (EAER)4 | km | 96 - 118 |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 Figures are preliminary and were determined internally in accordance with the “WLTP test procedure” certification method. At present, there has been neither an EC type approval nor a certificate of conformity with official values. There may be discrepancies between the figures provided and the official values. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 500 4MATIC Saloon, long version

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 330 [449]/5,500 – 6,100 |
| Rated torque1 | Nm at rpm | 600/2,200 – 5,000 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,921/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.90 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 530 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,195 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.5 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 9.1 - 8.3 |
| CO₂ emissions (combined)4 | g/km | 206 - 188 |
| CO₂ class4 |  | G |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 500 4MATIC Saloon

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 330 [449]/5,500 – 6,100 |
| Rated torque1 | Nm at rpm | 600/2,200 – 5,000 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.60 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 530 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,175 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.5 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 8.9 - 8.2 |
| CO₂ emissions (combined)4 | g/km | 203 - 187 |
| CO₂ class4 |  | G |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 The stated figures were determined in accordance with the prescribed WLTP (Worldwide harmonised Light vehicles Test Procedures) measuring process. The specified ranges refer to the European market. The energy consumption and CO₂ emissions of a passenger car depend not only on the efficient utilisation of the fuel or energy source by the vehicle, but also on the driving style and other non-technical factors. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 580 4MATIC Saloon, long version

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | V8 |
| Displacement | cc | 3,982 |
| Hub x bore | mm x mm | 92.0 x 83.0 |
| Rated output1 | kW [hp] at rpm | 395 [537]/5.500 - 6.100 |
| Rated torque1 | Nm at rpm | 750/2.500 – 4.500 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,921/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.90 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 530 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,270 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.0 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 11.3 - 10.6 |
| CO₂ emissions (combined)4 | g/km | 258 - 241 |
| CO₂ class4 |  | G |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 Figures are preliminary and were determined internally in accordance with the “WLTP test procedure” certification method. At present, there has been neither an EC type approval nor a certificate of conformity with official values. There may be discrepancies between the figures provided and the official values. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 580 4MATIC Saloon

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | V8 |
| Displacement | cc | 3,982 |
| Hub x bore | mm x mm | 92.0 x 83.0 |
| Rated output1 | kW [hp] at rpm | 395 [537]/5,500 – 6,100 |
| Rated torque1 | Nm at rpm | 750/2,500 – 4,500 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 17 [23] |
| Rated torque (peak) (electric) | Nm | 205 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.60 |
| Track width, front/rear | mm | 1,660/1,688 |
| Luggage compartment capacity, VDA2 | l | 530 |
| Kerb weight/payload/permissible gross mass3 | kg | 2,250 |
| Tank capacity/reserve | l | 76/8 |
| Permissible trailer load, unbraked, braked [at 12%] | kg | 750/2,100 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.0 |
| Top speed | km/h | 250 |
| Energy consumption (combined)4 | l/100 km | 11.2-10.5 |
| CO₂ emissions (combined)4 | g/km | 256 - 240 |
| CO₂ class4 |  | G |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 Figures are preliminary and were determined internally in accordance with the “WLTP test procedure” certification method. At present, there has been neither an EC type approval nor a certificate of conformity with official values. There may be discrepancies between the figures provided and the official values. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 580 e 4MATIC Saloon with EQ Hybrid technology, long version

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 330 [449]/5.500 - 6.100 |
| Rated torque1 | Nm at rpm | 560/2.200 – 5.000 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 120 [163] |
| Rated torque (peak) (electric) | Nm | 480 |
| Rated output system output | kW [hp] | 430 [585] |
| System torque | Nm | 750 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,304/1,921/1,503 |
| Wheelbase | mm | 3,216 |
| Turning circle | m | 11.90 |
| Track width, front/rear | mm | 1,655/1,683 |
| Luggage compartment capacity, VDA2 | l | 345 |

| Kerb weight/payload/permissible gross mass3 | kg | 2,515 |
| --- | --- | --- |
| Tank capacity/reserve | l | 67/8 |
| HV battery - usable energy content | kWh | 21.96 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.4 |
| Top speed | km/h | 250 |
| Energy consumption (weighted, combined)4 | kWh/100km plus l/100 km | 18.2–17.0 3.0-2.6 |
| Fuel consumption with discharged battery (combined)4 | l/100 km | 8.8–8.1 |
| CO₂ emissions (weighted, combined)4 | g/km | 73–61 |
| CO₂ class4 |  | B/G |
| Electric range (EAER)4 | km | 92-103 |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 Figures are preliminary and were determined internally in accordance with the “WLTP test procedure” certification method. At present, there has been neither an EC type approval nor a certificate of conformity with official values. There may be discrepancies between the figures provided and the official values. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.

Technical Data
Mercedes-Benz S 580 e 4MATIC Saloon with EQ Hybrid technology

As of: 11 December 2025

| Engine, drive system and battery |  |  |
| --- | --- | --- |
| Drive system |  | 4x4 (AWD) |
| Cylinder arrangement/number |  | L6 |
| Displacement | cc | 2,999 |
| Hub x bore | mm x mm | 92.4 x 83.0 |
| Rated output1 | kW [hp] at rpm | 330 [449]/5.500 - 6.100 |
| Rated torque1 | Nm at rpm | 560/2200 - 5000 |
| Fuel mix preparation |  | Injection pump/injection system model: Single plunger pump |
| Rated power (peak) (electric) | kW [hp] | 120 [163] |
| Rated torque (peak) (electric) | Nm | 480 |
| Rated output system output | kW [hp] | 430 [585] |
| System torque | Nm | 750 |
| Suspension and steering |  |  |
| Front axle |  | Front springs: Air spring |
| Rear axle |  | Rear springs: Air spring |
| Front brakes | mm | Brake type: Discs, internally ventilated and perforated Disc or drum diameter: 370 |
| Rear brakes | mm | Brake type: Discs, internally ventilated Disc or drum diameter: 360 |
| Steering |  | Power steering: Speed-dependent, electro-mechanical |
| Rims, front/rear |  | 8,5 J x 19 ET 31.5 / 8,5 J x 19 ET 31.5 |
| Tyres, front/rear |  | 255/45 R19 / 255/45 R19 |
| Dimensions and weights |  |  |
| Number of doors/seats |  | 4/5 |
| Length/width/height | mm | 5,194/1,921/1,503 |
| Wheelbase | mm | 3,106 |
| Turning circle | m | 11.60 |
| Track width, front/rear | mm | 1,655/1,651 |
| Luggage compartment capacity, VDA2 | l | 345 |

| Kerb weight/payload/permissible gross mass3 | kg | 2,495 |
| --- | --- | --- |
| Tank capacity/reserve | l | 67/8 |
| HV battery - usable energy content | kWh | 21.96 |
| Driving performance, consumption and range |  |  |
| Acceleration 0 - 100 km/h | s | 4.4 |
| Top speed | km/h | 250 |
| Energy consumption (weighted, combined)4 | kWh/100km plus l/100 km | 18.1–16.9 3.0-2.6 |
| Fuel consumption with discharged battery (combined)4 | l/100 km | 8.7–8.0 |
| CO₂ emissions (weighted, combined)4 | g/km | 71–60 |
| CO₂ class4 |  | B/G |
| Electric range (EAER)4 | km | 93-103 |

1 Figures for rated output and rated torque according to Regulation (EC) No. 595/2009 or Regulation (EC) No. 715/2007 in the currently applicable version.

2 according to Directive ISO 3832:2002-06. Boot capacity may differ depending on special equipment

3 Kerb weight, ready-to-drive in accordance with EU.

4 Figures are preliminary and were determined internally in accordance with the “WLTP test procedure” certification method. At present, there has been neither an EC type approval nor a certificate of conformity with official values. There may be discrepancies between the figures provided and the official values. Further information on the vehicles on offer, including the WLTP values, can be found on a country-specific basis at https://www.mercedes-benz.com.